# PART 1 – GENERAL

### 1.01	RELATED DOCUMENTS

The Pinellas County Schools Bidding and Contractual Requirements shall apply to all work.

## 1.02	DESCRIPTION OF WORK

Provide soil treatment for termite control as herein specified.

1.03	QUALITY ASSURANCE

A.	In addition to requirements of these specifications, comply with manufacturer’s instructions and recommendations for work, including preparation of substrate and application.

B.	The work under this section shall be performed by a Professional Pest Control Applicator, licensed in accordance with regulations of governing authorities for application of soil treatment solution.

1.04	JOB CONDITIONS

A.	Apply soil treatment pesticide solution only after excavating, filling and grading operations are completed, unless otherwise required in construction operations and with the owner’s approval.  Any violations of the chemical barrier after initial treatment due to excavation, moving pipes or conduit, etc., shall require immediate retreatment of affected area.

B.	To insure penetration, do not apply soil treatment to excessively wet soils or during inclement weather.  Comply with handling and application instructions of the soil toxicant manufacturer.

1.05	SUBMITTALS

Submit 3 copies of the manufacturer’s label, material safety data sheet (MSDS), and application instructions.

1.06	SPECIFIC PRODUCT WARRANTY

A.	Furnish written warranty certifying that applied soil poisoning treatment will prevent infestation of subterranean termites and, that if subterranean termite activity is discovered during warranty period, Contractor will re-treat soil and will repair or replace damage caused by termite infestation.

B.	Provide warranty, jointly signed by the Professional Pest Control Company Owner/Operator and the Contractor, for a period of ten years from the date of treatment.

PART 2 - PRODUCTS

2.01	CHEMICALS

A.	Use a water diluent, emulsible non-repellant concentrate insecticide specifically formulated to prevent termite infestation.  Tank used for application needs to be triple rinsed before use to ensure there is no chemical residue from repellant type insecticides.  Insecticides concentrate needs to be mixed in the tank by means of mechanical agitation to ensure proper and even dilution into water carrier.  Furnish and apply a working solution, mixed according to manufacturer’s instructions, from the following approved chemical concentrate and manufacturer:

Termidor 80WG or Termidor SC, mixed to .06% rate – BASF Corp.

B.	Only soil treatment solutions which are not injurious to planting are acceptable.  Fuel oil will not be permitted as a diluent.

C.	Other chemical manufacturers shall apply for approval as per the prior approval clauses within these specifications and shall show proof of acceptance by the local governing authorities and PCSB Pest Control Representative.

PART 3 - EXECUTION

3.01	GENERAL REQUIREMENTS

- Factory sealed, unopened containers of the pesticides intended for use on this project, undiluted and in its original formulation, shall be delivered to the construction site.  These containers shall only be opened and dilution or mixing of the pesticides shall only be performed by the licensed Pest Control Applicator at the construction site in the presence of the Pinellas County School Board (PCSB) Pest Control Representative.  It is the responsibility of the Contractor to notify the Architect and the PCSB Pest Control Representative (OFC 727-547-7219, Email:wormhoodt@pcsb.org) of the proposed dates and times set aside for soil treatment.

B.	Pesticides used for pre-construction soil treatment for the prevention of subterranean termites shall be applied under the direct supervision of a PCSB Pest Control Representative in a specific amount as designated by the manufacturer and in treatment areas as determined and approved by the Architect, PCSB Pest Control Representative, Contractor and the licensed Pest Control Applicator performing the soil treatment.  Every effort should be made to give a minimum of 3 business days notice of the treatment date.

C.	The PCSB Pest Control Representative reserves the right to halt soil treatment operation if the Representative determines:

1)	Intended specifications are not being adhered to.

2)	Weather conditions may be detrimental to the application of the pesticides.

3)	Barrier violation is occurring.

3.02	PREPARATION AND APPLICATION

A.	Surface Preparation:  Remove foreign matter which could decrease effectiveness of treatment on areas to be treated.  Loosen, rake, and level soil to be treated, except previously compacted areas under slabs and foundations.  Toxicants may be applied before placement of compacted fill under slabs, if recommended by toxicant manufacturer.

B.	Application:

1)	Apply soil treatment solution to indicated areas, and at rates specified.  Use strictly in accordance with manufacturer’s recommended mixing instructions.

2)	Areas and Rates:

a)	Under slab-on-grade structures, treat soil before concrete slabs are placed using either power sprayer or tank-type garden sprayer.

(1)	Apply four gallons of chemical solution per 10 lin. ft. to soil in critical areas under slab, including entire inside perimeter of foundation walls, along both sides of interior partition walls, around plumbing pipes and electric conduit penetrating slab, and around interior column foots.

(2)	Apply one gallon of chemical solution per 10 sq. ft. as an overall treatment under slab and attached slab areas where fill is soil or unwashed gravel.  Apply 1-1/2 gallons of chemical solution to areas where fill is washed gravel or other coarse absorbent material.

(3)	Apply four gallons of chemical solution per 10 lin. ft. of trench, for each of depth from grade to foot, along outside edge of building.  Dig a trench 6” to 8” wide along outside of foundation to a depth of not less than 12”, ensuring all construction debris has been removed from trench area and apply chemical solution.  Mix chemical solution with the soil as it is being replaced in trench.

b)	At hollow masonry foundations, treat voids at rate of two gallons, per 10 lin. ft., poured directly into the hollow spaces.

c)	At expansion joints, control joints, and areas where slabs will be penetrated, at rate of four gallons per 10 lin. ft. of penetration.

d)	Install vapor barrier immediately after chemical application is completed.

3)	Allow not less than 12 hours of drying after application, before beginning concrete placement or other construction activities.

a)	Protection:  Post signs in areas of application warning workers that soil poisoning has been applied.  Remove signs when areas are covered by other construction.

b)	Re-Application of Treated Area:  Reapply soil treatment solution to areas disturbed by subsequent excavation or other construction activities following application.

FOR IRRIGATION SECTION:  All irrigation heads will be placed a minimum of 12” away from all structures to prevent excessive moisture at base of structure.

FOR MASONRY SECTION:  All stucco or cementitious finish material shall be terminated at least 6” above final grade unless material is less than 5/8” thick and adhered directly to the masonry foundation sidewall.  If material is less than 5/8”, and finish is terminated below grade, a stop will be installed and termination line will be sealed with caulk of other material.

Any concrete over pour or mortar material accumulated along the exterior foundation perimeter shall be removed prior to exterior chemical soil treatment.

END OF SECTION